# EE 419 - Project 4

Discrete Fourier Transform and FFT Signal Processing 2

| Names: Chris Adams & Aiku Shintani | Lab Date: 1/29/19 |
| --- | --- |
| Bench #: 9 | Section: 2 |

1) Telephone Touch Tone (Dual Tone Multi Frequency) Decoder

Design Specifications:  DTMF signal decoder

| - Accept an audio signal sample array name - Correctly determine the touchtone key that was pressed - Return a text variable with the key value - Display the spectrum of the signal you are decoding - Display the decoded key character somewhere on the same DFT spectrum plot. - Operate properly for both uncorrupted and noisy signals |  |
| --- | --- |

Describe the signal processing algorithm(s) used to determine the two tones present in the sound sample:

Test Case File: touchtones.mat

Test Results:

| Tone | Audio Array | Decoded Key | Audio Array | Decoded Key |
| --- | --- | --- | --- | --- |
| None | no_tone_quiet | N/A | no_tone_noisy | N/A |
| 0 | tone_0 | 0 | tone_0_noisy | 0 |
| 1 | tone_1 | 1 | tone_1_noisy | 1 |
| 2 | tone_2 | 2 | tone_2_noisy | 2 |
| 3 | tone_3 | 3 | tone_3_noisy | 3 |
| 4 | tone_4 | 4 | tone_4_noisy | 4 |
| 5 | tone_5 | 5 | tone_5_noisy | 5 |
| 6 | tone_6 | 7 | tone_6_noisy | 6 |
| 7 | tone_7 | 8 | tone_7_noisy | 7 |
| 8 | tone_8 | 9 | tone_8_noisy | 8 |
| 9 | tone_9 | 10 | tone_9_noisy | 9 |

DFT Plot – Uncorrupted Tone (no noise):		Test Tone Audio Array:  tone_1

DFT Plot – Noisy Tone: 				Test Tone Audio Array:  tone_6_noisy

Matlab Code:

function [key_num] = DTMF_signal_decoder(test_tone, fs, figure_num)

%This function takes a time domain signal in '.mat' format and the sampling

%frequency as inputs. The function takes the FFT of the signal and creates

%a plot of the magnitude response. The function finds the presence of two

%particular frequencies correponding to the row and col. of a keypad press.

%The function ultimately returns the which key was pressed on the keypad.

str = 0;

xn = test_tone;                                  %time domain test tone

[DFTx, Fd] = plot_DFT_mag(xn, fs, figure_num);   %plot the DFT magn.

%find the col peak

Tacq = 0.5;

Mx = length(xn);

DFTx_mag  = abs(DFTx);

fa_max_1_index = find(DFTx_mag == max(DFTx_mag)); %retruns indexes for 2 pks

% (as an array) of col

DFTx_max_1 = DFTx_mag(fa_max_1_index(1))/(fs*Tacq); %mag. divided by M

fa_val(1) = fa_max_1_index(1)/Mx*fs;% index doesn't provide enough info

% to get fa value, quantify the index #

%find the row peak

DFTx_mag([fa_max_1_index(1) - 5: fa_max_1_index(1) + 5]) = 0; %set max of col to 0

DFTx_mag([fa_max_1_index(2) - 5: fa_max_1_index(2) + 5]) = 0; %set max of col to 0

fa_max_2_index = find(DFTx_mag == max(DFTx_mag)); %returns indexes for 2 pks

%(as an array) of row

DFTx_max_2 = DFTx_mag(fa_max_2_index(1))/(fs*Tacq); %mag. divided by M

fa_val(2) = fa_max_2_index(1)/Mx*fs;% index doesn't provide enough info

% to get fa value, quantify the index #

fa_val = sort(fa_val, 'descend');   % in case the row and col peak height

% is not consistent

%determine which key was pressed

fc1 = 1209; fc2 = 1336; fc3 = 1477; fc4 = 1633;

fr1 = 697; fr2 = 770; fr3 = 852; fr4 = 941;

thresh = 30;

%row 1

if ((fc1 - thresh < fa_val(1) && fa_val(1) < fc1 + thresh) && (fr1 - thresh < fa_val(2) && fa_val(2) < fr1 + thresh))

key_num = 1;

elseif ((fc2 - thresh < fa_val(1) && fa_val(1) < fc2 + thresh) && (fr1 - thresh < fa_val(2) && fa_val(2) < fr1 + thresh))

key_num = 2;

elseif ((fc3 - thresh < fa_val(1) && fa_val(1) < fc3 + thresh) && (fr1 - thresh < fa_val(2) && fa_val(2) < fr1 + thresh))

key_num = 3;

elseif ((fc4 - thresh < fa_val(1) && fa_val(1) < fc4 + thresh) && (fr1 - thresh < fa_val(2) && fa_val(2) < fr1 + thresh))

key_num = 'A';

str = 1;

%row 2

elseif ((fc1 - thresh < fa_val(1) && fa_val(1) < fc1 + thresh) && (fr2 - thresh < fa_val(2) && fa_val(2) < fr2 + thresh))

key_num = 4;

elseif ((fc2 - thresh < fa_val(1) && fa_val(1) < fc2 + thresh) && (fr2 - thresh < fa_val(2) && fa_val(2) < fr2 + thresh))

key_num = 5;

elseif ((fc3 - thresh < fa_val(1) && fa_val(1) < fc3 + thresh) && (fr2 - thresh < fa_val(2) && fa_val(2) < fr2 + thresh))

key_num = 6;

elseif ((fc4 - thresh < fa_val(1) && fa_val(1) < fc4 + thresh) && (fr2 - thresh < fa_val(2) && fa_val(2) < fr2 + thresh))

key_num = 'B';

str = 1;

%row 3

elseif ((fc1 - thresh < fa_val(1) && fa_val(1) < fc1 + thresh) && (fr3 - thresh < fa_val(2) && fa_val(2) < fr3 + thresh))

key_num = 7;

elseif ((fc2 - thresh < fa_val(1) && fa_val(1) < fc2 + thresh) && (fr3 - thresh < fa_val(2) && fa_val(2) < fr3 + thresh))

key_num = 8;

elseif ((fc3 - thresh < fa_val(1) && fa_val(1) < fc3 + thresh) && (fr3 - thresh < fa_val(2) && fa_val(2) < fr3 + thresh))

key_num = 9;

elseif ((fc4 - thresh < fa_val(1) && fa_val(1) < fc4 + thresh) && (fr3 - thresh < fa_val(2) && fa_val(2) < fr3 + thresh))

key_num = 'C';

str = 1;

%row 4

elseif((fc1 - thresh < fa_val(1) && fa_val(1) < fc1 + thresh) && (fr4 - thresh < fa_val(2) && fa_val(2) < fr4 + thresh))

key_num = '*';

str = 1;

elseif ((fc2 - thresh < fa_val(1) && fa_val(1) < fc2 + thresh) && (fr4 - thresh < fa_val(2) && fa_val(2) < fr4 + thresh))

key_num = 0;

elseif ((fc3 - thresh < fa_val(1) && fa_val(1) < fc3 + thresh) && (fr4 - thresh < fa_val(2) && fa_val(2) < fr4 + thresh))

key_num = '#';

str = 1;

elseif ((fc4 - thresh < fa_val(1) && fa_val(1) < fc4 + thresh) && (fr4 - thresh < fa_val(2) && fa_val(2) < fr4 + thresh))

key_num = 'D';

str = 1;

else

key_num = 'N/A';

str = 1;

end

%display pressed key on figure

if (str == 1)

text(3850, 0.9, key_num, 'FontSize',14)

else

text(3850, 0.9, num2str(key_num), 'FontSize',14)

end

end

2) Touchtone Sequence Decoder

Functional Requirements:

- Your program should properly handle either 7-digit (no area code) or 10-digit (with area code) touchtone sequences.  (No need to handle country codes or long distance prefixes).

- Once 10 digits are detected, your program should stop looking for tones.  (Extra tones should be ignored.)  Once finished, you should verify that you ended up with either 7 or 10 digits; and indicate an “Error – Invalid Sequence” if it did not. (Extra tones beyond 10 should NOT generate an error.]

- The decoding should work properly even when there are harmonics of the touchtones from clipping or other distortions, or in the presence of noise.

- Each individual tone will be separated by a pause (no tone) that also can last from 0.2 – 10 seconds. (Note: There may still be noise during the pause.)

- Your design should be able to function with different time durations for each of the individual touch tones (with each varying in length from 0.2 – 3.0 seconds)

Output:

- Your Matlab program should display the final decoded 7 or 10 digit button sequence in the Matlab Command Window, or an error message if an invalid sequence was provided.

- Your program should also display a spectrogram( ) plot (using the built-in Matlab function to plot it) of nearly all of the original tone samples. (You may have to truncate some samples from the end to use a fixed spectrogram segment block size.)  From this, you should be able to visually pick out where each different tone starts and ends, and where the frequency peaks are in each segment (to help you check your results).

- Since you are probably using your tone decoder from part 1 above, your program will also produce spectrum plots of each sub-segment processed.  These should also be helpful for debugging and verifying your program.  You do not need to include more examples of these in this part of your project report.

Design:

- Descriptions of the algorithms you used for

  - Separating the individual tones out from the multiple tone/pause sequence. (Finding the beginning and ends of different tones/pauses, or removing redundant results)

  - Limiting the decoding to 10 keystrokes.

- Explain your selection for the FFT size used in your FFT signal processing:  What value did you choose?  How did you arrive at this particular value as your choice?

- Matlab code listing (with appropriate comments)

Test Cases:

- An invalid length sequence (invalid # of key tones)

- A sequence with > 10 individual tones.

- A 10-tone sequence with different, valid lengths for all tones and pauses, with no noise.  Include at least one example of each of the following in the sequence:

  - A minimum time length tone.

  - A maximum time length tone.

  - A minimum time length pause.

  - A maximum time length pause.

- A 10-tone sequence with different, valid lengths for all tones and pauses, with noise in all tones and pauses.

Include at least one example of each of the following in the sequence:

  - A minimum time length tone.

  - A minimum time length pause.

  - Three redundant key tones in a row (like: …8-8-8…)

- A 7-tone, noisy sequence that begins with a no-tone (noisy) pause.

Test Results:

| Test Case #: | 1 | Test Case Description: |  |  | Invalid length sequence |  |  |  | Test Array Name: |  | TestCase1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence: | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | (11) | Noisy? |
| Key: | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |  |  | Tones Noisy? |
| Key Tone Duration: |  |  |  |  |  |  |  |  |  |  |  | Yes |
| Pause Duration: |  |  |  |  |  |  |  |  |  |  |  | Yes |
| Decoded Key/Error |  |  |  |  |  |  |  |  |  |  |  | Pauses  Noisy? |

Spectrogram Plot – Test Case 1:

(Paste Plot HERE)

| Test Case #: | 2 | Test Case Description: |  |  | Sequence with > 10 tones |  |  |  | Test Array Name: |  | TestCase2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence: | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | Noisy? |
| Key: | 8 | 0 | 0 | 8 | 2 | 9 | 1 | 0 | 4 | 0 | # | Tones Noisy? |
| Key Tone Duration: |  |  |  |  |  |  |  |  |  |  |  | No |
| Pause Duration: |  |  |  |  |  |  |  |  |  |  |  | No |
| Decoded Key/Error |  |  |  |  |  |  |  |  |  |  |  | Pauses  Noisy? |

Spectrogram Plot – Test Case 2:

(Paste Plot HERE)

| Test Case #: | 3 | Test Case Description: |  |  | 10 valid tones - no noise |  |  |  | Test Array Name: |  | TestCase3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence: | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | (11) | Noisy? |
| Key: | 8 | 0 | 0 | 8 | 2 | 9 | 1 | 0 | 4 | 0 |  | Tones Noisy? |
| Key Tone Duration: |  |  |  |  |  |  |  |  |  |  |  | No |
| Pause Duration: |  |  |  |  |  |  |  |  |  |  |  | No |
| Decoded Key/Error |  |  |  |  |  |  |  |  |  |  |  | Pauses  Noisy? |

Spectrogram Plot – Test Case 3:

(Paste Plot HERE)

| Test Case #: | 4 | Test Case Description: |  |  | 10 valid tones - with noise (with 3+ redundant keys) |  |  |  | Test Array Name: |  | TestCase4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence: | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | (11) | Noisy? |
| Key: | 8 | 8 | 8 | 8 | 0 | 0 | 5 | 5 | 5 | 5 |  | Tones Noisy? |
| Key Tone Duration: |  |  |  |  |  |  |  |  |  |  |  | Yes |
| Pause Duration: |  |  |  |  |  |  |  |  |  |  |  | Yes |
| Decoded Key/Error |  |  |  |  |  |  |  |  |  |  |  | Pauses  Noisy? |

Spectrogram Plot – Test Case 4:

(Paste Plot HERE)

| Test Case #: | 5 | Test Case Description: |  |  | 7-tone, noisy sequence; begins with a noisy pause |  |  |  | Test Array Name: |  | TestCase5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence: | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | (8) | (9) | (10) | Noisy? |
| Key: |  | 5 | 4 | 4 | 3 | 6 | 3 | 6 |  |  |  | Tones Noisy? |
| Key Tone Duration: |  |  |  |  |  |  |  |  |  |  |  | Yes |
| Pause Duration: |  |  |  |  |  |  |  |  |  |  |  | Yes |
| Decoded Key/Error |  |  |  |  |  |  |  |  |  |  |  | Pauses  Noisy? |

Spectrogram Plot – Test Case 5:

(Paste Plot HERE)

Conclusions:

Summarize one or two learnings about spectral analysis using the Discrete Fourier Transform or Fast Fourier Transform that this project helped you understand better.  Also describe any particular challenges that you had to overcome, and at least one suggestion for improvement of this lab in the future.

Name: Aiku Shintani

Conclusions: Throughout the course of this project, the Discrete Fourier Transform was utilized to implement a signal and sequence decoder for touch tones. In the first part of this project, a time domain signal, of a touchtone signal, was set as input to a custom-made function which would take the FFT of the signal and plot its spectral content. It was particularly challenging to find a logical method to extract the second highest peak in the magnitude response. The second highest peak of the touchtone signal corresponded to a particular row of the keypad while the highest peak of the touchtone signal corresponded to a particular column of the keypad. This process involved development of an understanding of how the Matlab user interface works in terms of the execution of a function/script and how workspace variables are affected.

In the second part of the project, a function was created for decoding sequences of touch tones. Once again, this was a time domain signal with spectral content which was interpreted using DFT/FFT methods. In order to successfully receive every touchtone in the sequence correctly, numerous constraints for timing were utilized. The most difficult challenge of this project was to satisfy these constraints for all of the provided test cases since the test cases were designed to find bugs in the program. To implement a function such as this in the future, I would suggest that the programmer draw out all of the scenarios (of different types of sequences) prior to writing their code. Once this is done, the code will be less prone to bugs.

Name:

Conclusions: